Solar Power Generation Forecasting Based on Regional Weather

Muhammad Miftah Faridh, Ida Bagus Krishna Yoga Utama, Radityo Fajar Pamungkas
Kookmin University.
Seoul, Republic of Korea
muhammadmiftahfaridh@gmail.com, idabaguskrishnayogautama@gmail.com, radityofajar@gmail.com, yjang@kookmin.ac.kr

Abstract— It will be impossible for humans to survive if they continue to rely on harmful fossil fuels; consequently, a renewable energy system is being created. Solar PV can satisfy the world's power demands, and by growing solar PV, we can estimate the electricity needs of a region in advance. Solar power production may be successfully predicted using forecasting models that use time series weather data. For ten areas in South Korea, we utilize LSTM, Bi-LSTM, Stacked Bi-LSTM, RNN, and the GRU model using solar generation and weather data. The best model was the stacked Bi-LSTM, which performed well in eight out of ten locations.

Keywords—Model, aaaa, bbbb, cccc

# Introduction

Because of the current reliance on fossil fuels, the human population will confront a catastrophic energy crisis [1]. Solar energy can considerably contribute to the world's electricity needs [2]. PV solar panels are mostly dependent on the regular pattern of solar irradiation. The radiation magnitude is zero at night, increases at dawn, and decreases at night. In addition, the weather conditions at certain times, which will block sun energy, will have an effect [3].

The time series model is one of this study's most commonly employed algorithms, although it has several distinct limitations. Due to this, numerous algorithms are employed in solar forecasting, including statistical time series models and artificial neural networks (ANNs), which have recently gained widespread acceptance for renewable energy forecasts due to their ability to self-learn and overcome the lack of time series [4]. LSTM, Bi-LSTM, Stacked Bi-LSTM, RNN, and GRU models were used to produce forecasts based on solar power generation and weather data from different regions in Korea.

# Methodology

Example of a figure caption. (figure caption)

To train a deep learning model, it is necessary to use data that aligns with the experiment's goals. Figure 1 depicts the data collection methods we employ. We collect PV sites in many regions of Korea, including Busan, Seoul, Gyeonggi, Gangwon, Daejeon, Gyeongnam, Jeonbuk, Sejong, Chungbuk, and Chungnam, for the production of solar power electrical statistics. The data is then sent to the database via MQTT. In addition, we require weather data to train the model; this data is available via the open API of the Korea Meteorological Administration (KMA). The data we collect is hourly data for the entire year of 2021.

# System Overview

Before you begin to format your paper, first write and save the content as a separate text file. Complete all content and organizational editing before formatting. Please note sections A-D below for more information on proofreading, spelling and grammar.

Keep your text and graphic files separate until after the text has been formatted and styled. Do not use hard tabs, and limit use of hard returns to only one return at the end of a paragraph. Do not add any kind of pagination anywhere in the paper. Do not number text heads-the template will do that for you.

## Data Preprocessing

After collecting data, it must be preprocessed due to the data's sensitivity to variations in power generation. We must interpolate some missing data using the nearest approach.- Then, the data are normalized using the MinMax method, the -1 to 1 rescaling approach. Then, we exclude features in the weather data by determining the best correlation between the data and the power generation data. Consequently, we only use data from meteorological inputs such as temperature, relative humidity, solar radiation, sunshine in seconds per hour, and precipitation per hour. To create a sequence-to-sequence, we turn the input into a sliding window format [5]. The sliding window data will consist of the previous month's and prior week's data for the future. Finally, we divide the data into three sections of corresponding sizes: training data, data validation data, and test data.

## Deep learning-based on Power Generation forecasting

This paper uses the deep learning model, such as LSTM, Bi-LSTM, Stacked Bi-LSTM, RNN, and GRU. They are all combined with a dense layer. In this experiment, we utilize several ANN models to identify compatibility with the input data, weather data. We set the batch size to 32, the hyperparameter Adam as the optimizer with a learning rate of 0.001, the iteration to 200, and the callback function to terminate if the data has already reached convergence or is close to overfitting.

## Deep learning-based on Power Generation forecasting

We present several findings regarding the predicted timeliness of time series data. Loss, validation loss, and mean square error (MSE) are the essential measures for evaluating a model's quality. The RMSE, the mean absolute error (MAE), and the regression score (R-squared or R2 score) were used to evaluate the accuracy of the predictions. We use the following equation to determine the error metrics.

# Experimental Result

Model Performance

| Region | Model | MSE | MAE | R2 Score |
| --- | --- | --- | --- | --- |
| Busan | LSTM | 0.0135 | 0.0556 | 0.9574 |
|  | Bi-LSTM | 0.0146 | 0.0564 | 0.9513 |
|  | Stacked Bi-LSTM | 0.014 | 0.0553 | 0.9560 |
|  | RNN | 0.0225 | 0.1007 | 0.9178 |
|  | GRU | 0.0171 | 0.0656 | 0.9459 |
| Seoul | LSTM | 0.0086 | 0.055 | 0.9256 |
|  | Bi-LSTM | 0.009 | 0.0529 | 0.9386 |
|  | Stacked Bi-LSTM | 0.0076 | 0.0385 | 0.9462 |
|  | RNN | 0.0161 | 0.0759 | 0.8966 |
|  | GRU | 0.0085 | 0.0479 | 0.9361 |
| Gyeonggi | LSTM | 0.0063 | 0.0406 | 0.9527 |
|  | Bi-LSTM | 0.007 | 0.0434 | 0.9488 |
|  | Stacked Bi-LSTM | 0.0062 | 0.0419 | 0.9433 |
|  | RNN | 0.0096 | 0.0672 | 0.8978 |
|  | GRU | 0.0058 | 0.0384 | 0.9503 |
| Gangwon | LSTM | 0.0145 | 0.0599 | 0.9465 |
|  | Bi-LSTM | 0.0178 | 0.0663 | 0.9246 |
|  | Stacked Bi-LSTM | 0.0125 | 0.0491 | 0.9534 |
|  | RNN | 0.0159 | 0.0661 | 0.9389 |
|  | GRU | 0.0143 | 0.0593 | 0.9466 |
| Daejon | LSTM | 0.0127 | 0.0593 | 0.9215 |
|  | Bi-LSTM | 0.0127 | 0.0596 | 0.9333 |
|  | Stacked Bi-LSTM | 0.0118 | 0.0584 | 0.9282 |
|  | RNN | 0.0169 | 0.0756 | 0.8798 |
|  | GRU | 0.0129 | 0.0589 | 0.9293 |
| Gyeongnam | LSTM | 0.0109 | 0.0543 | 0.939 |
|  | Bi-LSTM | 0.0104 | 0.0484 | 0.9459 |
|  | Stacked Bi-LSTM | 0.013 | 0.0593 | 0.9162 |
|  | RNN | 0.0121 | 0.0653 | 0.9269 |
|  | GRU | 0.0111 | 0.0548 | 0.9345 |
| Jeonbuk | LSTM | 0.0161 | 0.0618 | 0.9352 |
|  | Bi-LSTM | 0.0155 | 0.0587 | 0.9394 |
|  | Stacked Bi-LSTM | 0.0153 | 0.0558 | 0.9423 |
|  | RNN | 0.019 | 0.0753 | 0.9217 |
|  | GRU | 0.0161 | 0.0635 | 0.939 |
| Sejong | LSTM | 0.0159 | 0.0642 | 0.9409 |
|  | Bi-LSTM | 0.0155 | 0.0623 | 0.9442 |
|  | Stacked Bi-LSTM | 0.0148 | 0.0573 | 0.9471 |
|  | RNN | 0.0217 | 0.0788 | 0.9167 |
|  | GRU | 0.0157 | 0.0633 | 0.9463 |
| Chungbuk | LSTM | 0.0181 | 0.0714 | 0.885 |
|  | Bi-LSTM | 0.0182 | 0.0678 | 0.8867 |
|  | Stacked Bi-LSTM | 0.0176 | 0.0598 | 0.8982 |
|  | RNN | 0.0242 | 0.0815 | 0.8254 |
|  | GRU | 0.0185 | 0.0695 | 0.8869 |
| Chungnam | LSTM | 0.0099 | 0.0498 | 0.945 |
|  | Bi-LSTM | 0.0095 | 0.0478 | 0.9453 |
|  | Stacked Bi-LSTM | 0.0092 | 0.0441 | 0.9507 |
|  | RNN | 0.0113 | 0.06 | 0.9413 |
|  | GRU | 0.0104 | 0.0576 | 0.9368 |

The predictions made by five deep learning models and ten regions are displayed in Table 1. The table indicates that the stacked BiLSTM achieves the best performance in all regions. GRU is the second-best model for deep learning, followed by BiLSTM, LSTM, and RNN. Compared to other deep learning models, the stacked BiLSTM has the highest performance because of its architecture, which consists of many layers of BiLSTM.

# Conclusion

This work provides a comparison model employing 10 data regions in South Korea with fairly decent findings. Stacked Bi-LSTM with a single dense layer achieves excellent performance in eight out of ten regions, particularly in the MSE and MAE performance sections. In the future, we will investigate offering a technique that uses spatial-temporal analysis to produce new predictions using only data from the closest city.

##### Acknowledgment

This work was supported by the Technology Development Program (S3098815) funded by the Ministry of SMEs and Startups (MSS, Korea), and in part by the MSIT (Ministry of Science and ICT), Korea, under the ITRC (Information Technology Research Center) support program (IITP-2022-2018-0-01396) supervised by the IITP (Institute for Information & Communications Technology Planning & Evaluation).

##### References

V. Prema, M. S. Bhaskar, D. Almakhles, N. Gowtham, and K. U. Rao, “Critical review of data, models and performance metrics for wind and solar power forecast,” IEEE Access, vol. 10, pp. 667–688, 2022.

C.-H. Liu, J.-C. Gu, and M.-T. Yang, “A simplified LSTM neural networks for one day-ahead solar power forecasting,” IEEE Access, vol. 9, pp. 17174–17195, 2021.

J. R. Andrade and R. J. Bessa, “Improving renewable energy forecasting with a grid of numerical weather predictions,” IEEE Trans. Sustain. Energy, vol. 8, no. 4, pp. 1571–1580, 2017.

G. Li and J. Shi, “On comparing three artificial neural networks for wind speed forecasting,” Appl. Energy, vol. 87, no. 7, pp. 2313–2320, 2010.

- I. B. Krishna Yoga Utama, D. H. Tran, and Y. M. Jang, “Short-term PM2.5 prediction using modified attention Seq2Seq BiLSTM,” in 2022 Thirteenth International Conference on Ubiquitous and Future Networks (ICUFN), 2022